HAZMOVE SOFTWARE (ESP32)

System Overview & Communication Protocol

This technical report details the firmware integration on the ESP32 microcontroller enabling real-time wireless control of the HazMove Robot Arm via a Flutter mobile application. Communication is built upon the WebSocket protocol over Wi-Fi (operating on Port 81), providing bidirectional, low-latency data transfer. The ESP32 handles incoming JSON-formatted commands to actuate the servos and stepper motors, and broadcasts status updates encapsulating the physical state of all joints and links to synchronize the mobile application's 3D simulator.

1. Libraries, Constants, and Pin Configurations

#include <WiFi.h> #include <WebSocketsServer.h> #include <ArduinoJson.h> #include <ESP32Servo.h> #include <AccelStepper.h> #define WIFI_AP_MODE true const char* ssid = "HazMove_Robot_AP"; const char* password = "Password123"; WebSocketsServer webSocket = WebSocketsServer(81); #define BTN_SEQ1 34 #define BTN_SEQ2 35 #define PAUSE_SWITCH 14 #define BTN_STOP 12 #define SWITCH_REPEAT 13 #define LED_SEQ1 23 #define LED_SEQ2 22 Servo RobotUD1; // GPIO 19 (Base elevation) Servo RobotUD2; // GPIO 18 (Base elevation - inverted) Servo HandFB; // GPIO 5 Servo HandUD; // GPIO 15 Servo HandOC; // GPIO 21 Servo HandR; // GPIO 4 #define BASE_STEP 26 #define BASE_DIR 25 #define LIN_STEP 33 #define LIN_DIR 32 AccelStepper stepperBase(AccelStepper::DRIVER, BASE_STEP, BASE_DIR);

This section includes all critical libraries and hardware definitions. WiFi.h enables the wireless stack, WebSocketsServer.h handles TCP socket connections on port 81, and ArduinoJson.h manages JSON serialization and deserialization. The pins for servos, limit switches, buttons, and stepper drivers are declared. The ESP32 is configured to boot as an Access Point (WIFI_AP_MODE = true) creating a private local network, eliminating the need for external router infrastructure during standalone operation.

2. System States and Position Tracking

bool runningSequence = false; bool seq1Active = false; bool seq2Active = false; bool initialDone = false; bool forceStop = false; // Emergency stop flag bool softwarePause = false; // Remote pause flag float currentLinearCM = 0.0; float currentBaseDegrees = 0.0; unsigned long lastBlinkTime = 0; bool ledBlinkState = false; const unsigned long BLINK_INTERVAL = 200; const float STEPS_PER_DEGREE = 2.2222222222; const float STEPS_PER_CM = 2000; const int DEFAULT_BASE_SPEED = 1000;

State variables manage the operating mode of the robot. Global variables runningSequence, seq1Active, and seq2Active track automatic sequences. The forceStop boolean acts as a soft interlock for emergency shutdowns triggered via the app. softwarePause allows the mobile client to halt execution remotely. Position variables currentLinearCM and currentBaseDegrees maintain coordinates for the linear rail and base rotation, which are updated dynamically and fed back to the app's 3D simulator.

3. Logical-to-Physical Servo Mapping

void controlServoById(int id, int targetAngle, int speedDelay) { if (id == 1) { int start = HandFB.read(); MoveServo(HandFB, start, targetAngle, speedDelay); } else if (id == 2) { int targetScaled = map(targetAngle, 0, 180, 0, 100); static int currentArmAngle = 50; MoveArm(currentArmAngle, targetScaled, speedDelay); currentArmAngle = targetScaled; } else if (id == 3) { int start = HandUD.read(); MoveServo(HandUD, start, targetAngle, speedDelay); } else if (id == 4) { int start = HandR.read(); MoveServo(HandR, start, targetAngle, speedDelay); } else if (id == 5) { int start = HandOC.read(); MoveServo(HandOC, start, targetAngle, speedDelay); } }

The controlServoById function translates logical servo channels transmitted by the Flutter application (IDs 1-5) into the physical actuators on the robot. ID 1 moves the Forward/Backward hand servo. ID 2 controls the base elevation dual-servo pair (RobotUD1 & RobotUD2) using the custom MoveArm function which scales the 0-180 input down to a safe mechanical range of 0-100. IDs 3, 4, and 5 map to Hand Up/Down, Wrist Rotation, and Gripper Open/Close respectively.

4. WebSocket Event Callback and Command Parsing

void webSocketEvent(uint8_t num, WStype_t type, uint8_t * payload, size_t length) { switch(type) { case WStype_DISCONNECTED: Serial.printf("[%u] Disconnected!\n", num); break; case WStype_CONNECTED: { IPAddress ip = webSocket.remoteIP(num); Serial.printf("[%u] Connected from %s\n", num, ip.toString().c_str()); forceStop = false; sendStatusToClient(); break; } case WStype_TEXT: { StaticJsonDocument<256> doc; DeserializationError error = deserializeJson(doc, payload); if (error) return; const char* commandType = doc["type"]; if (strcmp(commandType, "servo") == 0) { int id = doc["servo_id"]; int targetAngle = doc["end"]; int speed = doc["speed"]; int speedDelay = map(speed, 1, 100, 30, 2); forceStop = false; controlServoById(id, targetAngle, speedDelay); } else if (strcmp(commandType, "linear") == 0) { double targetDistance = doc["distance"]; forceStop = false; double delta = targetDistance - currentLinearCM; if (delta != 0) { char dir = (delta > 0) ? 'R' : 'L'; MoveLinearCM(abs(delta), dir); } } else if (strcmp(commandType, "base") == 0) { double targetDegrees = doc["degrees"]; int speed = doc["speed"]; int stepperSpeed = map(speed, 1, 100, 100, 2000); forceStop = false; double deltaDegrees = targetDegrees - currentBaseDegrees; if (deltaDegrees != 0) { MoveBaseDegrees(deltaDegrees, stepperSpeed); } } else if (strcmp(commandType, "stop") == 0) { forceStop = true; stepperBase.stop(); sendStatusToClient(); } else if (strcmp(commandType, "pause") == 0) { softwarePause = true; sendStatusToClient(); } else if (strcmp(commandType, "resume") == 0) { softwarePause = false; sendStatusToClient(); } break; } } }

The webSocketEvent function operates as the asynchronous message broker. When a text frame containing JSON commands is received, deserializeJson decodes the message. The command is parsed dynamically. Servo commands translate speed metrics into step delay durations. Stepper movements compare targets against existing coordinate records to determine direction and count steps. Stop and Pause triggers override control loops to prevent motion immediately.

5. Dynamic Status Serialization & Feedback Broadcaster

void sendStatusToClient() { StaticJsonDocument<1024> doc; doc["type"] = "status"; JsonObject data = doc.createNestedObject("data"); JsonArray servos = data.createNestedArray("servos"); // Serialize Servo 1 (HandFB) JsonObject s1 = servos.createNestedObject(); s1["id"] = 1; s1["name"] = "Base Servo"; s1["currentAngle"] = HandFB.read(); s1["targetAngle"] = HandFB.read(); s1["speed"] = 50; s1["isMoving"] = false; // Serialize Servo 2 (RobotUD1 mapped back to 180 deg) JsonObject s2 = servos.createNestedObject(); s2["id"] = 2; s2["name"] = "Shoulder Servo"; s2["currentAngle"] = map(RobotUD1.read(), 0, 120, 0, 180); s2["targetAngle"] = map(RobotUD1.read(), 0, 120, 0, 180); s2["speed"] = 50; s2["isMoving"] = false; // ... (Other Servos 3, 4, 5 serialized here) JsonObject linearRail = data.createNestedObject("linearRail"); linearRail["currentPosition"] = currentLinearCM; linearRail["targetPosition"] = currentLinearCM; linearRail["speed"] = 50; linearRail["isMoving"] = false; JsonObject baseRotation = data.createNestedObject("baseRotation"); baseRotation["currentDegrees"] = currentBaseDegrees; baseRotation["targetDegrees"] = currentBaseDegrees; baseRotation["speed"] = 50; baseRotation["isMoving"] = false; data["mode"] = (seq1Active ¦¦ seq2Active) ? "auto" : "manual"; data["status"] = forceStop ? "error" : ((softwarePause ¦¦ digitalRead(PAUSE_SWITCH) == LOW) ? "paused" : (runningSequence ? "moving" : "idle")); data["isConnected"] = true; data["lastUpdated"] = "2026-06-30T18:05:25.000Z"; String output; serializeJson(doc, output); webSocket.broadcastTXT(output); }

The sendStatusToClient function formats and transmits state feedback. Using a 1024-byte static JSON document, it assembles data structures containing arrays for individual servos, linear rail status, and rotary platform alignment. The current position values of physical stepper drivers are tracked, and servos are polled using .read() to generate accurate currentAngle values. The resulting JSON payload matches the structure expected by the mobile client's data models exactly, enabling real-time telemetry.

6. Multi-tasking & Non-blocking Management

bool KeepAlive() { webSocket.loop(); PauseButton(); if (forceStop) { return false; } return true; } void PauseButton() { while (digitalRead(PAUSE_SWITCH) == LOW ¦¦ softwarePause) { webSocket.loop(); PauseLED(); } if (seq1Active) digitalWrite(LED_SEQ1, HIGH); if (seq2Active) digitalWrite(LED_SEQ2, HIGH); }

The KeepAlive function prevents networking timeouts during long hardware movements. By inserting KeepAlive() inside blocking movement loops (e.g., servo sweeps or stepper pulse generation), the firmware ensures webSocket.loop() is polled at high frequency. If the mobile app issues a STOP payload, KeepAlive() detects forceStop = true, returning false to abort the enclosing loop immediately. Similarly, PauseButton() continuously processes WebSockets during pause conditions so the connection remains active and a resume packet can be received.

7. Actuator Motion Control Logic

void MoveServo(Servo &servo, int startAngle, int endAngle, int speedDelay) { if (startAngle > endAngle) { for (int a = startAngle; a >= endAngle; a--) { if (!KeepAlive()) break; servo.write(constrain(a, 0, 180)); delay(speedDelay); } } else { for (int a = startAngle; a <= endAngle; a++) { if (!KeepAlive()) break; servo.write(constrain(a, 0, 180)); delay(speedDelay); } } sendStatusToClient(); } void MoveLinearCM(float cm, char direction) { long steps = cm * STEPS_PER_CM; if (direction == 'L') { digitalWrite(LIN_DIR, LOW); currentLinearCM -= cm; } else if (direction == 'R') { digitalWrite(LIN_DIR, HIGH); currentLinearCM += cm; } else return; for (long i = 0; i < abs(steps); i++) { if (!KeepAlive()) break; digitalWrite(LIN_STEP, HIGH); delayMicroseconds(200); digitalWrite(LIN_STEP, LOW); delayMicroseconds(200); } sendStatusToClient(); }

This block handles physical motor drive routines. MoveServo implements incremental rotation adjustments with interleaved KeepAlive calls to provide controlled velocity profiles. MoveLinearCM converts target metrics (in cm) into step outputs at 2.5 kHz (pulse period of 400 microseconds). Both functions publish state packets immediately upon target completion to guarantee visual synchronization with the frontend.

8. Initialization and Execution Poll

void setup() { Serial.begin(115200); // Wi-Fi initialization... webSocket.begin(); webSocket.onEvent(webSocketEvent); // Input/Output pins configuration... RobotUD1.attach(19); RobotUD2.attach(18); HandFB.attach(5); HandUD.attach(15); HandOC.attach(21); HandR.attach(4); stepperBase.setMaxSpeed(1000); stepperBase.setAcceleration(500); } void loop() { webSocket.loop(); if (!initialDone) { InitialPosition(); initialDone = true; } // Hardware buttons polling... static unsigned long lastStatusTime = 0; if (millis() - lastStatusTime > 1000) { lastStatusTime = millis(); sendStatusToClient(); // Periodic status report } }

The setup function configures hardware peripherals, links the servos to GPIOs, sets stepper driver limits, and binds the WebSocket event callbacks. The main loop executes webSocket.loop() constantly to receive packets. It polls hardware buttons (BTN_SEQ1 and BTN_SEQ2) for manual sequence triggers, and issues a 1Hz telemetry heartbeat status report to verify network connection and update status fields when idle.